10 Krey Boulevard  Rensselaer, NY  12144

March 19, 2014

By Electronic Delivery

Ms. Kimberly D. Bose, Secretary

Federal Energy Regulatory Commission 888 First Street, NE

Washington, DC 20426

Re:    New York Independent System Operator, Inc., Docket ER08-1281-___;

Broader Regional Markets Informational Report and Request to Modify the Reporting Obligation

Dear Ms. Bose:

In accordance with paragraph 33 and ordering paragraph “D” of the Federal Energy 
Regulatory Commission’s (“Commission’s” or “FERC’s”) December 30, 2010 Order on 
Rehearing and Compliance in Docket No. ER08-1281 (“December 2010 Order”),1 the New 
York Independent System Operator, Inc. (“NYISO”) hereby submits this Report on Broader 
Regional Markets (“Report”).  Ordering paragraph “D” of the December 2010 Order states that 
the “RTO/ISO parties are hereby directed to submit informational reports, as discussed in the 
body of this order.”  In footnote 35 of its December 2010 Order the Commission stated that 
it does not intend to issue public notice or issue an order on this Informational Report.

Paragraph 33 of the December 2010 Order instructs the NYISO:

“in collaboration with its neighboring RTO/ISOs, NERC and other 
market participants, to submit a report, as an information filing, 
addressing (i) the effects of the reforms on reducing congestion 
that results from loop flows and the costs associated with 
mitigating congestion; (ii) the effects of the implementation of the 
enhanced interregional transaction coordination initiative; and (iii) 
recommendations and analyses as to whether the buy-through 
congestion proposal is required, and if so, when it should be 
implemented.35”

35 These reports will be for informational purposes only.  They will not be noticed and the Commission does not intend to act on them.

1 New York Independent System Operator, Inc., 133 FERC ¶ 61,276 (2010).

Ms. Kimberly D. Bose, Secretary March 19, 2014

Page 2

While the NYISO is responsible for submitting this informational Report to the

Commission, the contents of the Report were developed through collaboration between and

among PJM Interconnection, LLC (“PJM”), the Midcontinent Independent System Operator, Inc. (“MISO”), the Ontario Independent Electricity System Operator (“IESO”) and the NYISO 
(collectively the “Lake Erie ISOs and RTOs”), with input from the NYISO’s stakeholders and the North American Electric Reliability Corporation (“NERC”).

I.	Improvements that the Lake Erie ISOs and RTOs have made since 2008

A.	Market Design Improvements the ISOs/RTOs Have Completed or Are

Presently Developing

The Lake Erie ISOs and RTOs present the following summary of the market design

features that have been, or will be, implemented to improve coordination between markets and reduce Lake Erie loop flow.

1.	NYISO’s Interface Pricing Improvements

In December 2010, the Commission instructed the NYISO to develop interface pricing 
reforms to address the price incentives that create loop flow concerns.2  The NYISO’s interface 
pricing rules, as accepted by the Commission in 2013,3 incorporate four key aspects.  First, the 
NYISO models the Michigan/Ontario interface in the same way PJM does.  Second, the NYISO 
includes expected unscheduled power flows (“UPF”) in both its Day-Ahead Market (“DAM”) and 
Real-Time Market (“RTM”).  Third, the NYISO uses its scheduling path validation process to 
ensure that External Transaction Bids are economically evaluated and scheduled consistent with 
their expected power flow impacts.4 Finally, to calculate prices at its Keystone Proxy Generator 
Bus that represents a significant portion of the facilities that interconnect the New York Control 
Area (“NYCA”) to PJM, the NYISO (a) treats uncontrolled alternating current (“A/C”)

transmission lines as free-flowing tie lines, (b) includes expected (Day Ahead) or actual (realtime) UPF over the interface facilities in its pricing, and (c) recognizes the demonstrated 
intermediate- and long-term effectiveness of the ABC, JK and Ramapo Phase Angle Regulators (“PARs”) in aligning actual power flows with scheduled power flows on PAR controlled 
transmission facilities at the PJM/NYCA border.  The NYISO’s interface pricing rules also 
incorporate Commission-accepted tariff obligations and contractual obligations into the NYISO’s development of its Keystone Proxy Generator Bus prices.

2 New York Independent System Operator, Inc., 133 FERC ¶ 61,276 at PP 27 and 31 (2010).

3 New York Independent System Operator, Inc., 142 FERC ¶ 61,202 (2013).

4 Dr. Patton, President of the NYISO’s Market Monitoring Unit, explained why it is appropriate for the NYISO to

continue to use its bid validation software: “The NYISO’s path validation process is designed to ensure that the actual 
power flows associated with the transactions are as consistent with the scheduled flows as possible. Precluding 
circuitous paths substantial reduces unscheduled loop flows and reduces market participants’ ability engage in 
patterns of transactions that may constitute manipulation of the RTO’s interface pricing.”  See NYISO’s January 18, 
2013 Interface Pricing Compliance Filing in Docket Nos. ER08-1281 and ER13-780, Attachment I at P 16.

Ms. Kimberly D. Bose, Secretary March 19, 2014

Page 3

The NYISO’s interface pricing policy relies on its scheduling path validation

process.5  NYISO Bid validation occurs as soon as a Bid is submitted to the NYISO’s Market 
Information System (“MIS”), and before Bids are made available to be economically evaluated 
for scheduling by the NYISO’s Day-Ahead or Real-Time Market software.  The NYISO’s Bid 
validation only allows feasible transactions that contain valid NERC e-Tag data, to be 
economically evaluated for possible scheduling.  The NYISO’s Bid validation software will not 
validate Bids submitted to schedule External Transactions over any of the eight circuitous 
Prohibited Transmission Paths identified in Section 16.3.3.8 of (Attachment J to) the NYISO 
Open Access Transmission Tariff (“OATT”).  The Commission has instructed the NYISO to add 
additional prohibited paths if necessary to prevent new sources of Lake Erie loop flow from 
being introduced in the future.6

The Commission found that the NYISO’s interface pricing policy satisfies the

requirements to align scheduled and real-time energy flows and to utilize NERC eTag validation 
to enhance the alignment of scheduled and actual power flows.  NYISO accounts for expected 
unscheduled power flows in its Day-Ahead Market based on recently observed Lake Erie 
circulation. “In the real-time market, NYISO accounts for the difference between scheduled and 
actual power flows measured at its interface with Ontario, i.e., actual Lake Erie circulation.”7

The NYISO’s interface pricing rules produce prices that are consistent with the value of 
actual power deliveries to New York and are similar to the prices produced by PJM’s pricing 
method.  Greater consistency in regional pricing methods, combined with NYISO’s rules that 
prohibit the use of circuitous scheduling paths have diminished both the ability and the 
incentives for Market Participants scheduling External Transactions to cause Lake Erie loop 
flow.  Instead, more power is being scheduled consistent with the path over which it will actually 
flow.

2. 	PJM/NYISO Market-to-Market Implementation

The Commission authorized the NYISO and PJM to begin coordinated congestion

management/market-to-market coordination (“M2M”) in January 2013.8  The M2M coordinated 
congestion management process allows transmission constraints that are significantly impacted 
by generation dispatch changes in both the NYISO and PJM markets or by the operation of the 
Ramapo PARs to be jointly managed in the real-time security-constrained economic dispatch 
models of both RTOs.9  This joint real-time management of transmission constraints near the

5 New York Independent System Operator, Inc., 142 FERC ¶ 61,202 at P 26 (2013).

6 Id. at P 27.

7 Id. at P 22.

8 New York Independent System Operator, Inc., 138 FERC ¶ 61,192 (2012).

9 NYISO/PJM M2M includes two types of coordination: (1) re-dispatch coordination; and (2) Ramapo PAR

coordination.  For re-dispatch coordination, the non-monitoring RTO re-dispatchs its generation to help manage congestion in the monitoring RTO, when economic, if one of the pre-defined flowgates becomes congested in the monitoring RTO.  For Ramapo PAR coordination, the Ramapo PARs are operated to reduce overall congestion if certain pre-defined flowgates become congested in one or both RTOs.

Ms. Kimberly D. Bose, Secretary March 19, 2014

Page 4

market borders provides a more efficient and lower cost transmission congestion management solution, and facilitates price convergence at the market boundaries.  Real-time coordination results in a more efficient economic dispatch solution to manage the real-time transmission constraints that impact both markets.

In its Orders accepting the rules NYISO and PJM developed to implement M2M, the

Commission explained “the Market-to-Market Coordination Process was developed specifically 
in response to problems associated with Lake Erie loop flows.”10  The Commission Order 
accepting the M2M revisions to the parties’ Joint Operating Agreement concluded “that 
coordinating the redispatch of resources and PAR operations in both NYISO and PJM as 
compared to each market operating independently, and thereby limiting the set of resources 
available for redispatch to resources located within the control area where the constraint is 
located, is a preferred method for addressing interregional transmission constraints and the Lake 
Erie loop flows.”11

The Lake Erie ISOs and RTOs have learned to proactively address possible new causes 
of Lake Erie loop flow.  In February 2013, one of the two Ramapo PARs at the PJM/New York 
border experienced an unplanned forced outage.  To address the expected impact of the PAR 
outage on their transmission systems, interchange scheduling practices, and implementation of 
M2M coordination, the NYISO and PJM requested a temporary waiver from their Commission 
accepted JOA provisions to permit them to operate their systems in a way that better reflected 
expected power flows during the Ramapo PAR outage.12  The waiver request allowed NYISO

and PJM to maximize the total interchange capability between the RTOs while one PAR was out of service and to continue to achieve the target interchange flow over the Branchburg - Ramapo (5018) A/C transmission line (the “5018 line”).13  Specifically, NYISO and PJM requested 
permission to temporarily reduce the net scheduled interchange that is expected to flow over the 5018 line, from 61 percent to 46 percent, while the PAR was being replaced.  The waiver applied to the finite period of May 15, 2013 until the Ramapo PARs were returned to normal operation. NYISO and PJM are dedicated to proactively addressing interregional coordination issues as they arise, and involving the Commission when necessary, to more efficiently manage 
interregional transmission constraints and the Lake Erie loop flow.

Potomac Economics, the NYISO’s external market monitoring unit, (“MMU”) has been 
reviewing M2M operations between NYISO and PJM.14  The MMU regularly analyzes both the 
efficacy of re-dispatch coordination and the efficacy of Ramapo PAR coordination.  In its most 
recent quarterly report, the MMU “finds that the Ramapo Line was reasonably efficient in 72

10 New York Independent System Operator, Inc., 140 FERC ¶ 61,205 at P 22 (2012).

11 New York Independent System Operator, Inc., 138 FERC ¶ 61,192 at P 20 (2012).

12 New York Independent System Operator, Inc., 143 FERC ¶ 61,153 at P 5 (2013).

13 The Ramapo PARs are utilized to control flows, and achieve target flow, over the 5018 line between NYISO and 
PJM.

14 See Quarterly Report on the New York ISO Electricity Markets Third Quarter 2013, slides 61-68,

http://www.nyiso.com/public/webdocs/markets_operations/documents/Studies_and_Reports/Reports/MMU_Quarter 
ly_Reports/2013/NYISO%20Quarterly%20Report%20-%20Quarter%203.pdf.

Ms. Kimberly D. Bose, Secretary March 19, 2014

Page 5

percent of the hours with congestion in NY and/or PJM.”  During this period, M2M re-dispatch 
coordination was not utilized often enough to allow reporting of that components efficacy.  The 
MMU presents this quarterly review to New York Market Participants at stakeholder meetings.15

The M2M coordination process has provided NYISO and PJM a set of very effective 
tools to assist their management of congestion caused by unscheduled power flows including 
Lake Erie loop flow.  During the first year of implementation, 2013, the NYISO estimated that 
the value of NYISO/PJM M2M was $4.7M.16  The estimate represents the value New York 
realizes from Ramapo PAR coordination.  This includes (1) the estimated savings to NYISO for 
additional deliveries into New York, and (2) PJM compensation to NYISO for additional 
deliveries into PJM (as compared to the Ramapo Target Value,17 excluding service to RECO 
load).  The identified value is net of any settlements to PJM when PJM’s transmission system is 
congested.

3. 	Implementation of More Frequent Scheduling

Enhanced Interregional Transaction Coordination (“EITC”) permits the scheduling of 
inter-Balancing Authority transactions on a more frequent basis than hourly schedules.18

PJM, MISO and NYISO are all capable of scheduling interregional interchange on a 15-
minute basis.  MISO and PJM implemented the ability to schedule interregional interchange on a 
15-minute basis on May 1, 2004.  On July 27, 2011, NYISO activated 15-minute scheduling at 
its Chateauguay D/C interface with Hydro-Quebec.  In 2012 NYISO activated 15-minute 
scheduling at all of its interfaces with PJM, including all Scheduled Lines.19  When Transmission 
Customers offer interregional energy on a 15-minute basis, the RTO/ISO dispatchers have the 
ability to alter schedules within an hour to address changing system conditions, including 
changes in unscheduled power flows like Lake Erie loop flow.  This dispatching flexibility 
allows the RTO/ISOs to respond to the price impacts that Lake Erie loop flow has on their 
transmission systems by altering intra-hour external transaction schedules.  The RTO/ISO 
dispatchers are no longer locked into achieving hourly external transaction schedules.

15 The most recent presentation is available at the following link:

http://www.nyiso.com/public/webdocs/markets_operations/documents/Studies_and_Reports/Reports/MMU_Quarter 
ly_Reports/2013/NYISO%20Quarterly%20Report%20-%20Quarter%203.pdf.

16 http://www.nyiso.com/public/webdocs/markets_operations/committees/mc/meeting_materials/2014-01-
29/Operations_Report_201312.pdf

17 The Ramapo Target Value is defined in the NYISO/PJM JOA, Schedule D, Section 7.2.1.  The Ramapo Target Value is based on the net interchange schedule between the NYISO and PJM plus the deviation of actual flows and desired flows across the ABC and JK interfaces.

18 New York Independent System Operator, Inc., 134 FERC ¶ 61,186 (2011).

19 The NYISO and PJM activated 15-minute scheduling on their primary interface on June 27, 2012.  Fifteen minute scheduling was implemented on the Neptune and Linden VFT scheduled line interfaces on October 30, 2012 and November 28, 2012, respectively.

Ms. Kimberly D. Bose, Secretary March 19, 2014

Page 6

In its order accepting the NYISO’s EITC proposal, the Commission recounted that 
enhanced scheduling options at the NYISO’s borders promote more efficient interregional 
transmission, reduce uplift costs associated with real-time event management and congestion 
management, support system balancing efforts by expanding the pool of resources available to 
system operators, and lower total system operating costs by improving price signals.20  Potomac 
Economics’ analysis identified a potential annual production cost savings for all NYISO 
interfaces of $175 million.21

The NYISO has made use of the additional flexibility that 15-minute scheduling

provides.  The NYISO varied external transaction schedules at its PJM Keystone Proxy

Generator Bus in over 33% of all quarter hours in 2013.22  Since implementation, the NYISO has consistently received 15-minute transaction offers at four of the five interfaces that allowed 15-
minute scheduling in real-time.23

Moving forward, the NYISO will focus on improving the 15-minute scheduling protocol 
by implementing Coordinated Transaction Scheduling (“CTS”) with both PJM and ISO-New 
England (“ISO-NE”) as well as evaluating the feasibility of 5-minute scheduling with Hydro-
Quebec.

4.	PJM/NYISO and ISO New England/NYISO Coordinated Transaction

Scheduling

The CTS Real-Time Market rules, recently accepted by the Commission,24 allow Imports 
and Exports to be scheduled based on a bidder’s willingness to purchase energy at a source in 
one Control Area and sell it at a sink in another Control Area if the forecasted price at the sink 
minus the forecasted price at the corresponding source is greater than or equal to the bid dollar 
value.25  The NYISO intends to implement CTS with PJM in November of 2014 and CTS with 
ISO-NE in the fourth quarter of 2015.  CTS between NYISO and PJM will be implemented at all 
four of the Proxy Generator Buses over which interchange between the two areas can be

scheduled.

20 New York Independent System Operator, Inc., 134 FERC ¶ 61,186 at P 8 (2011).

21 Id.

22 33% is a measure of schedule changes across the xx:15, xx:30 and xx:45 quarter hours.  The xx:00 time was

excluded because changes across xx:00 can occur due to changes in offer sets and would not provide evidence of improved inter-regional scheduling.

23 The fifth interface is a Scheduled Line where the capability has been pre-sold to anchor tenants that choose to schedule on an hourly basis.

24 See New York Independent System Operator, Inc., 139 FERC ¶ 61,048 (2012) and New York Independent System Operator, Inc., 146 FERC ¶ 61,097 (2014).

25 Transmission Customers using CTS will submit a single CTS Interface Bid to indicate their desire to

simultaneously buy Energy in one Control Area and sell Energy into the other Control Area based on the forecasted price difference between the NYISO and PJM markets at the relevant location.

Ms. Kimberly D. Bose, Secretary March 19, 2014

Page 7

As the Commission recognized in its order accepting the NYISO’s CTS with PJM filing, “CTS will enhance market efficiency of interregional transactions and provide substantial 
benefits to consumers in both PJM and NYISO.”  “CTS should also minimize counter intuitive 
flows, such as flows going from a high priced control area to a low priced control area, by 
incorporating projected price differences between the NYISO and PJM markets into scheduling decisions.”  In addition, “CTS should improve scheduling efficiency for both regions by 
introducing a new scheduling option… that will allow bidding of different MW quantities at 
different prices for each 15 minute interval within an hour.”26  CTS will further improve 
scheduling efficiency for transactions between NYISO and PJM by establishing intra-hour 
schedules 15 minutes closer to actual, real-time operations.  Establishing intra-hour schedules 
closer to the actual 15 minute scheduling interval will improve the accuracy of cross-border 
scheduling decisions because those decisions will reflect updated system conditions, including 
any price impact of unscheduled Lake Erie loop flow.27

The Commission found that both CTS with PJM and CTS with ISO-NE will provide 
substantial benefits to customers in the three affected regions.  In its CTS with PJM order, the 
Commission finds “that CTS will enhance market efficiency of interregional transactions and 
provide substantial benefits to consumers in both PJM and NYISO … joint studies performed by 
PJM and NYISO estimate potential production cost savings ranging from $9 million/year to $26 
million/year.”28  The order accepting the NYISO’s CTS with ISO-NE filing similarly states, 
“CTS will provide substantial benefits to consumers in both ISO-NE and NYISO by addressing 
inefficiencies present in the current external transaction scheduling process …, for the combined 
ISO-NE and NYISO region, Potomac Economics estimates that CTS will result in $129 million 
to $139 million in annual consumer savings, and $9 million to $11 million in annual production 
cost savings.”29

5.	PJM/MISO Interchange Optimization

PJM and MISO began discussions on the development of an Interchange Optimization

solution for stakeholder consideration as part of the MISO-PJM Joint and Common Market effort 
to address seams issues on the MISO-PJM interface.  Current plans call for the development of a 
proposal to be presented to stakeholders for their endorsement in the Summer 2014 time frame, 
with their approval being sought in the Fall 2014 followed by a FERC filing in early 2015.

B. 	Physical Improvements Implemented Since 2008

As of April 5, 2012, all four circuits comprising the Michigan/Ontario interconnection 
had in-service PARs.  Starting on that date, the MISO and IESO began actively operating the 
PARs to better conform actual power flows to scheduled power flows.  The expectation was that

26 New York Independent System Operator, Inc., 146 FERC ¶ 61,097 at P 33 (2014).

27 Id. at P 13.

28 Id. at P 33.

29 New York Independent System Operator, Inc., 139 FERC ¶ 61,048 at P 29 (2012).

Ms. Kimberly D. Bose, Secretary March 19, 2014

Page 8

such operations, in conjunction with controls already operational elsewhere on the system, would help reduce the unscheduled flows which cause Lake Erie Circulation (“LEC”).

In January 2014, MISO, PJM and IESO completed an evaluation of the PARs on the

Ontario-Michigan interface and their ability to maintain actual flow within a 200 MW bandwidth 
of scheduled flow and produced an Evaluation Report.30  Based on an analysis using one year of 
actual operating data when all PARs on the Ontario-Michigan interface were operational, the 
evaluation found the PARs are able to keep Lake Erie loop flow within a ±200 MW control band 
during 73.1% of the 15-minute periods during the one-year study period.  The simulated loop 
flow calculated without PAR control would only have been within the control band for 43.4% of 
the year.  During most of the periods that the loop flow strayed outside the 200 MW bandwidth, 
the flow was over by a small margin and the flow was expected to return within the 200 MW 
bandwidth within the next few 15-minute periods.  MISO and IESO set the Ontario-Michigan 
interface status to “Regulated” 95.5% of the time during the study period.

The purpose of the Evaluation Report was to recount the actual performance of the 
Ontario-Michigan (ONT-MI) interface PARs over a one-year period during which the PARs 
were operational, and to provide insight into the effectiveness of the PARs in controlling LEC 
flow.  The Evaluation Report follows from the Regional Power Control Device Coordination 
(“RPCDC”) Study report published in 2011 as a joint effort among IESO, MISO, NYISO, and 
PJM.

Although the RPCDC Study recommended a follow-up study (Second Study) be performed after the Ontario-Michigan PARs enter service and operational data had been collected for a year, the Evaluation Report should not be considered as meeting that recommendation for three reasons:

•   First, one of the Ramapo PARs was out-of-service from February 2013 until late-
	December 2013.  Since the one-year of operating data used in this analysis (8/1/12-
	7/31/13) contains seven months during which there was not a fully functioning set of 
	PARs on the PJM-NYISO interface, the data does not support an analysis on how the 
	various power control devices around Lake Erie influence LEC or could have their 
	operations coordinated to minimize loop flows.

•   Second, rather than defer the study until one year of operating data with a fully

functioning set of PARs on the PJM-NYISO interface is available, the study to support 
the Evaluation Report was performed in 2013 to address a Joint and Common Market 
(JCM) initiative that MISO/PJM evaluate the ability of the Ontario-Michigan PARs to 
manage LEC by having actual flow equal scheduled flow.  If the Ontario-Michigan PARs 
are effective in managing LEC, the JCM initiative will recommend they be included in 
the MISO/PJM market flow calculations and in the historic allocation process.

30 The Ontario-Michigan PAR Performance Evaluation Report (“Evaluation Report”) is posted at http://www.miso-
pjm.com/documents.aspx.

Ms. Kimberly D. Bose, Secretary March 19, 2014

Page 9

•   Third, NYISO elected not to participate in this study and Evaluation Report but did 
	participate in the first RPCDC Study.  NYISO has indicated that it is willing to 
	participate in a second study at some point in the future, provided that actual operating 
	data includes the impacts of all Ramapo PARs.  Therefore, this study should be 
	considered a limited scope study that addressed a specific JCM initiative.  A second study 
	is still planned for the future and is anticipated to include the involvement of all four 
	RTOs/ISOs around Lake Erie.

Section I.B. of this Report was prepared by MISO, PJM and IESO.  The NYISO does not join in this section of the Report.31

II.	The Lake Erie ISOs and RTOs Agree That Implementing Buy-Through of

Congestion is Premature at This Time

The objective of the proposed Buy-Through of Congestion Broader Regional Market

solution is to (a) identify the sources of loop flow caused by interregional transaction scheduling,

(b) determine the costs incurred in supporting the loop flows by each impacted region, and

(c) allocate the costs incurred by the off-contract path Balancing Authorities to the scheduling 
entity, or remove the associated schedules if the scheduling entity is not willing to pay the full 
cost of flowing its transaction(s).  Implementing Buy-Through of Congestion will result in a 
more complete identification, and accurate assignment, of the costs to move power between 
regions, and will provide an economic alternative to the administrative/physical curtailment 
processes.  Buy-Through of Congestion will allow the scheduling entity to decide whether or not 
it is willing to pay the congestion charges caused by its transaction’s off-contract path flow 
impacts.  If a scheduling party indicates it is not willing to pay congestion charges, its transaction 
will be removed if the off-contract path flow impacts add to congestion costs in an off-contract 
path ISO or RTO.

The Lake Erie ISOs and RTOs need time to understand how the collective set of market 
solutions discussed herein will affect Lake Erie loop flow and how the market solutions should 
account for the operation of all of the controllable devices around Lake Erie.  The only way to 
understand the impact of these market solutions is to gain operational experience with the 
recently, and soon to be, implemented Broader Regional Market improvements and to study the 
resulting operational data.  NYISO, PJM, MISO and IESO all agree that it is not necessary to 
begin to develop the proposed Buy-Through of Congestion Broader Regional Market solution at

31 The NYISO does not possess the necessary information and has not been given the opportunity to analyze and

substantiate MISO’s and IESO’s claims.  All of the Lake Erie ISOs and RTOs agree that it would be counter-

productive for NYISO and PJM to address the operation of the Michigan/Ontario PARs in this informational Report. On October 20, 2010, MISO and International Transmission Company d/b/a ITCTransmission (“ITC”) submitted a Federal Power Act Section 205 filing to the Commission, in Docket No. ER11-1844, to allocate a portion of the cost of ITC’s Bunce Creek PARs (two of the five Michigan/Ontario PARs) to customers in New York and PJM.  On 
December 20, 2011, the FERC Chief Administrative Law Judge set Docket No. ER11-1844 for hearing before an Administrative Law Judge.  The presiding Administrative Law Judge issued a post hearing initial decision on 
December 18, 2012, Midwest Indep. Transmission Sys. Operator, Inc., 141 FERC ¶ 63,021 (2012), which remains pending before the Commission.  All of the hearing parties (including MISO, NYISO and PJM) are waiting for a Commission order on the Administrative Law Judge’s initial decision.

Ms. Kimberly D. Bose, Secretary March 19, 2014

Page 10

this time.  The Lake Erie ISOs and RTOs need more time to analyze the other recently, and soon to be, implemented programs before determining whether or not the Buy-Through of Congestion Broader Regional Market solution will provide sufficient additional benefits to merit its 
development and implementation.

III. 	Request for Waiver or Modification of Reporting Obligation

The December 2010 Order imposed semiannual reporting obligations on the NYISO, in collaboration with its neighboring ISOs and RTOs, NERC and other market participants, 
commencing one year after the implementation of interface pricing reform and congestion 
management/market-to-market coordination.

“[O]ne year after the implementation of interface pricing reform 
and congestion management/market-to-market coordination, and 
every six months thereafter until the market initiatives are fully 
implemented, we require the NYISO, in collaboration with its 
neighboring RTO/ISOs, NERC and other market participants, to 
submit a report, as an information filing, addressing: (i) the effects 
of the reforms on reducing congestion that results from loop flows 
and the costs associated with mitigating congestion; (ii) the effects 
of the implementation of the enhanced interregional transaction 
coordination initiative; and (iii) recommendations and analyses as 
to whether the buy-through congestion proposal is required, and if 
so, when it should be implemented.”32

The ISOs and RTOs respectfully request that the Commission amend this informational filing obligation to require annual reports to the Commission.  Postponing the next informational filing to March 20, 2015 will provide necessary time for the ISOs and RTOs to develop a more 
substantive update for the Commission than could be provided in September 2014.  Certain 
initiatives described above (CTS, for example) will require the majority of 2014 to complete and, therefore, the status will not change significantly in the next six months.  Also, an additional year of operating experience with the features that entered service in 2012-2013, and the features 
being implemented in 2014, will allow the ISOs and RTOs to better understand their impact 
before providing an update to the Commission.

IV. 	Communications and Correspondence

All communications and service in this proceeding should be directed to:

32 New York Independent System Operator, Inc., 133 FERC ¶ 61,276 at P 33 (2010) (original footnotes omitted).

Ms. Kimberly D. Bose, Secretary March 19, 2014

Page 11

Robert E. Fernandez, General Counsel

Raymond Stalter, Director, Regulatory Affairs

*Alex M. Schnell, Registered Corporate Counsel *James H. Sweeney, Attorney

New York Independent System Operator, Inc.

10 Krey Boulevard

Rensselaer, NY 12144 
Tel:  (518) 356-6000 
Fax: (518) 356-8825

aschnell@nyiso.com 
jsweeney@nyiso.com

* Designated to receive service.

V.	Service

The NYISO will send an electronic link to this filing to the official representative of each 
party to this proceeding, to the official representative of each of its customers, to each participant 
on its stakeholder committees, to the New York Public Service Commission, and to the New 
Jersey Board of Public Utilities.  In addition, the complete filing will be posted on the NYISO’s 
website at www.nyiso.com.

VI.	Conclusion

The NYISO respectfully requests that the Commission accept this Report as satisfying the requirements set forth in the Commission’s December 2010 Order and accept the proposed annual reporting requirement that is proposed in Section III of this Report.

Respectfully submitted,

/s/ Alex M. Schnell

Alex M. Schnell, Registered Corporate Counsel James H. Sweeney, Attorney

New York Independent System Operator, Inc.

cc:	Michael A. Bardee

Gregory Berson

Anna Cochrane

Jignasa Gadani

Morris Margolis

Michael McLaughlin 
David Morenoff 
Daniel Nowak

CERTIFICATE OF SERVICE

I hereby certify that I have this day served the foregoing document upon each person

designated on the official service list compiled by the Secretary in this proceeding in accordance 
with the requirements of Rule 2010 of the Rules of Practice and Procedure, 18 C.F.R. § 
385.2010.

Dated at Rensselaer, NY this 19th day of March, 2014.

By:	/s/ John C. Cutting

John C. Cutting

New York Independent System Operator, Inc.

10 Krey Blvd.

Rensselaer, NY 12144 (518) 356-7521