UNITED STATES OF AMERICA 
	BEFORE THE

FEDERAL ENERGY REGULATORY COMMISSION

New York Independent System Operator, Inc.	)	Docket No. AD14-8-000

POST-TECHNICAL CONFERENCE COMMENTS

OF THE NEW YORK INDEPENDENT SYSTEM OPERATOR, INC.

In accordance with the Commission’s March 19, 2014 Supplemental Notice of Technical Conference (“Supplemental Notice”) in this proceeding, the New York Independent System Operator, Inc. (“NYISO”) respectfully submits its post-technical conference comments.  These comments discuss the impacts of recent cold weather events on the NYISO and discuss actions taken by the NYISO to prepare for and respond to those cold weather occurrences.

I. 	NYISO WINTER 2013-2014 MARKET OPERATIONS

Winter 2013-2014 was characterized by numerous periods of colder than average weather 
beginning in December 2013 and extending into February 2014.  The winter included five major 
cold snaps, including three polar vortexes that extended across much of the United States.  On 
January 7, 2014, the NYISO set a new record winter peak load of 25,738 MW during a polar 
vortex.

The NYISO met all reserve requirements throughout the winter operating period, despite 
significant generator capacity derates on some of the coldest days.  The NYISO did not activate 
emergency procedures at any time during the winter operating period, although the NYISO

called demand response resources and issued public appeals for customers to curtail or reduce non-essential electric power use on January 7, 2014.

On several days during winter 2013-2014 the price of natural gas exceeded the price of 
oil, making oil-fired generators, including dual-fuel generators running on oil, more economic 
than natural gas-fired generators.  In January 2014, the load-weighted price of electricity was 
$183/MWh, a 176% increase compared to the December 2013 price of electricity.  Also in 
January 2014, the average price of natural gas was $27.43/mmbtu, indexed at Transco Zone 6 in 
the New York City area, almost 400% larger than the December 2013 price of natural gas.  The 
NYISO’s efficient selection of economic dispatch is demonstrated by a percentage increase in 
the price of electricity that was less than half of the natural gas price increase. 
	The NYISO Operations Department (“Operations”) undertook two distinct efforts in 
response to the winter 2013-2014 cold weather market conditions.  During the early part of the 
winter, Operations’ efforts focused on managing generator capacity derates, some of which 
resulted from fuel supply or cold weather issues.  NYISO also worked with the New York 
Transmission Owners (“NYTOs”) to cancel or reschedule transmission outages.  During the later 
part of the winter, due to the increased use of oil-fired resources, Operations’ efforts focused on 
monitoring potential fuel depletions that could lead to generator capacity derates.

II. 	NYISO WINTER 2013-2014 OBSERVATIONS

Generators with confirmed gas nominations were generally successful in generating to 
their day-ahead electric commitments despite the numerous cold weather periods, including 
during gas pipeline Operational Flow Orders (“OFOs”) or System Alerts.  Generators connected 
to the interstate pipelines were also able to procure and nominate gas intraday, at times during

2

the winter, including for NYISO Supplemental Resource Evaluation (“SRE”) commitments (e.g., additional generators committed for reliability to meet changed or local system conditions). 
	The ability to purchase and nominate gas intraday helped maintain bulk power system and local Transmission Owner system reliability; however, gas purchasing flexibility is not 
available at all times of the day, even though generators may have the option to nominate gas 
throughout the day depending on the pipeline (i.e., additional nomination cycles outside of the 
standard intra-day cycles).  There was at least one occasion this winter when a generator could 
not respond to a late-evening requested reliability commitment due to fuel limitations, but could respond the following day.  While some pipelines allow for hourly nominations, it is the 
NYISO’s understanding that gas marketers are not always available outside of normal business 
hours for a generator to arrange a gas purchase.

Pipeline OFOs and System Alerts were in effect many days during the winter of 2013-
2014. OFOs that limit daily balancing tend to be less restrictive than OFOs that require hourly 
balancing.  Daily balancing restrictions usually require a generator to stay within its scheduled 
gas nominations, limiting overburn to a specific threshold over the course of the gas day.  Hourly 
flow restrictions that require a generator to stay within its scheduled gas nominations each hour 
are more restrictive and limit generator flexibility to respond to electric system conditions.  The 
NYISO observed both types of OFOs this winter; however, hourly balancing restrictions were 
generally applied only on the gas local distribution company (“LDC”) systems.

In preparing for the 2013-2014 winter, the NYISO implemented new cold weather

procedures to monitor gas nominations, oil inventories, and expected oil replenishment schedules for all dual-fuel, gas-fired, and oil-fired generators prior to each cold day.  This near real-time monitoring is in addition to an annual survey the NYISO conducts each fall that requests

3

information on gas transportation arrangements, oil inventory, and oil replenishment capability from the same generators.  The NYISO uses this fuel information to monitor the generators’ capability to meet Day-Ahead electricity schedules.

III. 	NYISO LESSONS LEARNED AND NEXT STEPS

The NYISO is exploring potential market design changes to address two reliability

concerns observed during the sustained cold weather of winter 2013-2014: (i) the significant

number of generator capacity derates on cold days; and (ii) the potential for limited fuel supplies 
during sustained cold periods.  The NYISO will continue to perform its seasonal fuel 
assessments of gas transportation, alternative fuel inventories and predicted replenishment rates. 
In addition, the NYISO is exploring ways to improve generator energy offer reference level 
management during time periods when the gas prices can change rapidly or when more frequent 
fuel type changes may be necessary (e.g., natural gas to oil or oil to natural gas). 
	The NYISO is also working to increase its gas system awareness by incorporating the fuel capability of units into the visual displays in the Operations’ control room.  The ability to monitor the actual fuel capability of units (gas, oil, etc.), especially during periods of the more restrictive hourly OFOs, will provide additional real-time information to help NYISO Operations maintain electric system reliability.  The NYISO is also coordinating efforts with the interstate, intrastate, and LDC pipelines to incorporate gas pipeline outage scheduling with NYISO electric system outage scheduling.

The NYISO is also actively participating in the Eastern Interconnection Planning

Collaborative (“EIPC”) Gas-Electric System Interface Study (“EIPC Study”).  The EIPC Study, 
funded by the U.S. Department of Energy, is evaluating the interaction between the natural gas 
and electric systems over the next five and ten years.  The EIPC Study covers a broad study

4

region, consisting of the service territories served by NYISO, PJM Interconnection L.L.C., ISONew England, Independent Electricity System Operator (Ontario), Midcontinent Independent System Operator, Inc. and Tennessee Valley Authority.  Future NYISO planning efforts will be informed by the EIPC Study results.

IV.	QUESTION OF COMMISSIONER PHILIP MOELLER ON COLD WEATHER

TECHNICAL CONFERENCE

Question:  For each power plant that ran this winter, and that is expected to retire due to EPA regulations, what resources will replace it?

Answer: The NYISO does not own or develop capacity resources.  Generation is

developed by Market Participants as they deem it economic to do so, with the resources that they 
choose.  The NYISO is not aware of any generators that operated during the winter 2013-2014 
and have announced their intention to retire or mothball unit(s) as the direct result of EPA 
regulations.

The NYISO Interconnection Queue specifies what generation Market Participants are 
developing.  As of April 2014, more than 5,000 MW of proposed new gas-fired generation is 
listed in the NYISO Interconnection Queue.  Approximately 4,240 MW of the proposed gas-
fired generators are combined cycles, some of which will have dual-fuel capability.  The 
remaining resources in the NYISO Interconnection Queue are gas-only combustion turbines. 
	The NYISO conducts an annual assessment of the reliability of the New York State Bulk Power Transmission Facilities in accordance with criteria, rules and procedures established by the NYISO, North American Electric Reliability Corporation (NERC) Reliability Standards, Northeast Power Coordinating Council (NPCC), and New York State Reliability Council 
(NYSRC).  In addition, as part of its tariff-based Comprehensive Reliability Planning Process, 
the NYISO conducts a Reliability Needs Assessment (“RNA”) that provides a long-range

5

reliability assessment of both resource adequacy and transmission security of the New York bulk 
power system over a ten-year study period.  The RNA considers any units that have submitted a 
notice of their intent to retire or mothball prior to April 15 of the planning year as retired for the 
period of the study.  Proposed new generation projects (as listed in the NYISO interconnection 
queue or as planned and listed in the NYISO Gold Book), and projects proposed as market-based 
solutions by Transmission Owners also may be considered for inclusion in the RNA model.

If the long-range reliability assessment shows issues surrounding resource adequacy or 
transmission security (“Reliability Need”), the NYISO will request market-based and alternative 
regulated proposals from interested parties and designate one or more Responsible Transmission 
Owners to develop a Regulated Backstop Solution to address each identified need.  Should it 
appear that market-based solutions will not meet the Reliability Need by the need date, the 
NYISO can trigger a Regulated Backstop Solution or an Alternative Regulated Solution to 
proceed to seek regulatory approvals and build its project.  Should the NYISO determine that an 
imminent threat to bulk power system reliability will arise, the NYISO may also request that a 
Transmission Owner or Developer seek approval of a Gap Solution to meet the reliability need.

V.	CONCLUSION

In conclusion, the NYISO respectfully submits these post-technical conference comments for the Commission’s consideration.

Respectfully submitted,

/s/   James H. Sweeney

James H. Sweeney, Attorney

New York Independent System Operator, Inc.

May 15, 2014

6

cc:	Michael Bardee

Gregory Berson

Anna Cochrane

Jignasa Gadani

Jesse Hensley

Morris Margolis

Michael McLaughlin 
David Morenoff 
Daniel Nowak 
Adria Woods

7