Elizabeth A. Grisaru 
	Of Counsel

518.487.7624 phone

egrisaru@woh.com

December 28, 2010

By Electronic Filing

Honorable Kimberly D. Bose, Secretary Federal Energy Regulatory Commission 888 First Street, NE

Washington, DC 20426

Re:    New York Independent System Operator, Inc., Docket No. ER11-xxxx-xxx, 
	Proposed Tariff Revisions to Support Enhanced Interregional Transaction 
	Coordination

Dear Secretary Bose:

In accordance with Section 	205 of the Federal Power Act and Part 	35 of the

Commission’s regulations, the New York Independent System Operator, Inc. 	(“NYISO”)

respectfully submits proposed revisions to its Market Administration and Control Area Services 
Tariff (“Services Tariff”) and Open Access Transmission Tariff (“OATT”).1  If approved, these 
proposals will allow the NYISO to implement more frequent transaction scheduling at the 
borders of the New York Control Area (“NYCA”), which will enable system operators to utilize 
transmission between New York and its neighbors more efficiently. The NYISO anticipates that 
these measures will result in reduced uplift costs associated with real-time event management 
and congestion management, facilitate better integration of intermittent resources, and lower 
total system operating costs.  The proposed amendments received extensive stakeholder review 
over the past twelve months, and obtained approvals from the NYISO’s Management Committee 
and the NYISO’s independent Board of Directors in October and November 2010, respectively.

1 Capitalized terms not specifically defined herein have the meaning set forth in the Services Tariff and the OATT.

Honorable Kimberly D. Bose December 28, 2010

Page 2

Part II of this filing describes the background and purpose of the proposed tariff revisions.  Part III discusses the specific OATT and Services Tariff changes that are needed to implement the NYISO’s proposals.

I.	Documents Submitted

1.    This filing letter;

2.  A clean version of the proposed revisions to the Services Tariff and the OATT 
	(“Attachment I”);

3.  A black lined version of the proposed revisions to the Services Tariff and the OATT 
	(“Attachment II”); and

II.	Background to this Filing

In its January 12, 2010 compliance filing in the ER08-1281 docket, the NYISO proposed to 
pursue both physical and market solutions to the problem of Lake Erie loop flows.2  With the 
assistance and support of the Midwest Independent Transmission System Operator, Inc. 
(Midwest ISO”), PJM Interconnection, LLC (“PJM”), the Ontario Independent System Operator 
(“IESO”), and their stakeholders, the NYISO’s compliance filing recommended four market 
solutions:   Buy-Through   of   Congestion,   Congestion   Management/Market   to   Market 
Coordination, Interface Pricing Revisions, and Interregional Transaction Coordination. The 
Commission accepted the NYISO’s filing, conditioned on the filing of answers to certain 
questions.3  The NYISO filed its responses on August 16, 2010 and reported that it was 
continuing to work collaboratively with IESO, Midwest ISO, PJM, ISO New England (“ISO-
NE”), and Hydro Quebec Trans Energie (“HQ”) to develop the concepts it had proposed.4

With the present filing, the NYISO submits the tariff revisions that are necessary to 
implement one of those recommended market solutions, Enhanced Interregional Transaction 
Coordination, in New York.  These provisions will put in place the program the NYISO and its 
partners contemplated in the January 12 filing and report by offering Market Participants the

2 New York Independent System Operator, Inc., Report on Broader Regional Markets, Docket No. ER08-1281 (January 12, 2010).

3 New York Independent System Operator, Inc., 132 FERC ¶ 61,031, at 11 (2010).

4 New York Independent System Operator, Inc., Response to Questions and

Supplemental Report on Broader Regional Markets, Docket No. ER08-1281, at 3 (August 16, 
2010).

Honorable Kimberly D. Bose December 28, 2010

Page 3

ability to schedule flexible transactions at the NYISO’s external interfaces.  The NYISO 
anticipates that implementation of this capability will lower total system operating costs by 
improving the consistency of transaction schedules with market-to-market prices, expanding the 
pool of resources available to the system operator to balance intermittent resources, improving 
transmission  utilization,  and  reducing  the  uncertainties  associated  with  forward-looking 
scheduling horizons.

There are two fundamental elements to the NYISO’s proposal.  First, the tariffs must be revised to authorize more frequent scheduling of External Transactions.  Second, the pricing rules that currently apply at external Proxy Generator Buses need to be modified to support flexible scheduling.  The modified pricing rules must also take into account the fact that, as the Commission has previously recognized, some external interfaces are not fully competitive.5  The NYISO further proposes to clarify the existing import curtailment guarantee settlement rules for all Proxy Generator Buses, which the NYISO will apply independently of the flexible scheduling and revised pricing changes described here.6

The NYISO’s Real-Time Market rules and Real Time Commitment (“RTC”) and Real Time Dispatch (“RTD”) software presently allow Market Participants to schedule energy at the NYCA borders only once per hour.  The NYISO settles those hourly transactions at the applicable realtime clearing prices.  Today, there is a time gap of 60 to 120 minutes between scheduling the energy and the determination of the real-time prices at which the transactions are settled.  This gap exposes traders to locked-in positions and potential volatility in the real-time market prices. Reducing the time between the scheduling and pricing of energy at the NYCA border will reduce this pricing risk and should ultimately increase efficiency and decrease costs to consumers in New York and in neighboring Control Areas.7

5 New York Independent System Operator, Inc., 104 FERC ¶61,220 (2003); 105 FERC ¶61,347 (2003) (accepting tariff revisions recognizing certain external proxy buses as noncompetitive).  To date, Proxy Generator Buses associated with the HQ Control Area and the Dennison Scheduled Line have been designated as non-competitive.  The NYISO is not 
proposing to change those designations here.

6 Import curtailment guarantees are intended to protect a Market Participant’s Day Ahead settlement in the event of a real-time curtailment imposed by the NYISO for reliability.  The 
proposed clarifications implement that principle in language that will apply to hourly as well as intra-hour Transactions.

7 An analysis prepared by Potomac Economics, the NYISO’s independent Market

Monitor, suggests a total annual production cost savings for all NYISO interfaces of $175

million as a result of adopting the more frequent external transaction scheduling protocols

proposed here.  Analysis of the Broader Regional Markets Initiative, Potomac Economics

presentation to Joint NYISO-IESO-MISO-PJM Stakeholder Technical Conference on Broader Regional Markets (September 27, 2010), available at

Honorable Kimberly D. Bose December 28, 2010

Page 4

The tariff changes discussed below will provide the NYISO and Market Participants the ability to schedule energy at designated Proxy Generator Buses on a fifteen-minute basis, and in some instances, on a five-minute basis.  At the same time, Market Participants will continue to have the ability to schedule hourly transactions at those Proxy Generator Buses, at their option. However, until the NYISO implements flexible scheduling at all Proxy Generator Buses, wheelthrough transactions will be unable to opt for intra-hour scheduling, in order to ensure matching injection and withdrawal schedules.

This filing also proposes revised pricing rules to be implemented in conjunction with the 
introduction of intra-hour scheduling at any particular Proxy Generator Bus.8  The pricing rules 
vary depending on whether (1) the NYISO has designated the Proxy Generator Bus as available 
for flexible scheduling, (2) the same bus has also been designated as non-competitive or is 
associated with a designated Scheduled Line, and (3) the bus is or is not subject to a constraint.9 
The NYISO will continue to utilize the RTC and RTD software to schedule transactions and 
evaluate price constraints.

All External Transactions at a given Proxy Generator Bus will settle based on the LBMPs 
derived by RTD unless the Proxy Generator Bus is constrained.  In such a case, Transactions at 
the constrained location will settle using the appropriate alternative pricing rule.  The proposed 
tariff sheets identify the various formulas that the NYISO will use depending on scheduling 
frequency and system conditions at any given Proxy Generator Bus; in general, the proposed 
rules require the NYISO to substitute more appropriate LBMPs for the RTD LBMPs when 
congestion or other constraints limit either imports or exports at that location.10  The substitute 
LBMPs will include the constraint costs at the Proxy Generator Buses resulting from the 
economic evaluation and scheduling of external transactions, where the LBMP outputs from 
RTD may not include these costs, as external transaction schedules may have already been 
established prior to the RTD execution.

If the Proxy Generator Bus is non-competitive, or if it is associated with a designated 
Scheduled Line, the NYISO will apply a different set of rules to determine pricing.  Again, the

http://www.nyiso.com/public/webdocs/committees/bic_miwg/meeting_materials/2010-09-
27/BRM_Analysis_Presentation_to_RTOs_9-27-10.pdf.

8 More details of the Proxy Generator Bus Pricing Rules can be found at

http://www.nyiso.com/public/webdocs/committees/bic_miwg/meeting_materials/2010-06-
07/Agenda_04_EITC_Scheduling_Pricing_Concept.pdf.

9 The designated Scheduled Lines are the Cross Sound Cable, the Linden VFT, and the Neptune line.  Services Tariff, Attachment B, section 17.1.6.3.  A specific Proxy Generator Bus is associated with each of these for purposes of transaction scheduling.

10 Special pricing rules will also apply to Transactions at the designated Scheduled Lines.

Honorable Kimberly D. Bose December 28, 2010

Page 5

choice of pricing rule is dictated by whether or not the particular bus is available for hourly or 
more flexible scheduling and by the presence or absence of specific constraints.  As the 
Commission has noted, LBMPs at a non-competitive facility or designated Scheduled Line can 
be influenced by limited opportunities for competition during constrained periods. Thus 
additional rule sets need to be applied to mitigate the possibility of non-competitive outcomes.

The Commission should note that these revised pricing rules are conceptually the same as 
those employed by the NYISO today at Proxy Generator Buses.  The modifications are designed 
to extend the current Commission-approved pricing to accommodate the various scheduling 
frequencies that will be possible as the NYISO implements intra-hour scheduling at the NYCA 
borders.  The NYISO’s pricing will continue to substitute LBMPs that reflect the value of 
constraints for those generated by RTD.  Where an interface is non-competitive, the pricing will 
continue to recognize minimum and maximum bounds, as the current rules do today.

The proposal also includes modifications to the eligibility rules for real-time bid production 
cost guarantees for transaction bids at Proxy Generator Buses available for more flexible 
scheduling; these are intended to create an incentive to offer intra-hour transactions in real-time.

The NYISO intends to implement the proposed more frequent scheduling and new pricing 
rules one Proxy Generator Bus at a time.  The NYISO will provide at least a two week notice to 
its Market Participants before activating more frequent scheduling at a Proxy Generator Bus. 
Those Proxy Generator Buses that have not yet been designated as available for intra-hour 
scheduling will continue to be governed by the existing pricing and bid production cost 
settlement rules.

III. 	Description of Tariff Changes

Implementation of the NYISO’s proposals requires modifications to both the Services Tariff and the OATT.  The principal changes are described below.

A.  Changes to Definitions.  The tariff sheets submitted with this filing include changes 
	to the definitions of certain terms that appear in both tariffs, as well as new terms.

Services Tariff 2.4, OATT 1.4:  A change to the definition of Desired Net Interchange will facilitate more frequent External Transaction scheduling.

The NYISO also proposes to add the term “Dynamically Scheduled Proxy Generator 
Bus”  to  identify  those  locations  at  which  the  NYISO  will  be  capable  of 
accommodating transactions that are subject to modification every five minutes in 
real-time.

Services Tariff 2.5, OATT 1.5:  The new term “Energy Profile MW” will be used to specify the maximum desired schedule for an External Transaction Bid.

Honorable Kimberly D. Bose December 28, 2010

Page 6

Services Tariff 	2.18, OATT 1.18:  The NYISO proposes minor changes to the 
definitions of “Real-Time Commitment” or “RTC” and “Real-Time Dispatch”

(“RTD”) to authorize intra-hour scheduling at a Proxy Generator Bus.  The NYISO is also adding a new term, “Rolling RTC,” to identify any RTC run that schedules fifteen-minute External Transactions.

Services Tariff	2.19, OATT 1.19:  The NYISO proposes modifications to the

definition of	“Sink Cap Price Bid” that will allow for bid curves on Export

transactions.

Services Tariff 2.22, OATT 1.22:  The NYISO is introducing another new term, 
“Variably Scheduled Proxy Generator Bus.”  This term identifies locations at which 
the NYISO will offer to schedule External Transactions at fifteen minute intervals.

B. 	Services Tariff Changes.  The NYISO proposes several changes to Sections 4.4.1,

4.4.2, 4.4.3, and 4.5 of the Services Tariff that are necessary to implement intra-
hourly transactions.  The NYISO is also proposing related changes to Attachments B, 
C, and J.

Section 4.4.1.1:  The NYISO revises this section to clarify that RTC will produce binding schedules for External Transactions on a quarter-hour basis.

Section 	4.4.1.2.2:  The changes proposed for this section eliminate the fixed 
requirement that External Transaction Bids have a minimum one-hour duration and 
substitutes provisions that will allow the NYISO to implement variable scheduling at 
specific Proxy Generator Buses with the agreement of the parties submitting the Bids.

Section 4.4.1.3:  The NYISO’s revisions will allow RTC to schedule intra-hour External Transactions by the quarter hour at Proxy Generator Buses for which variable scheduling has been authorized.  The changes also provide that the amount of Energy scheduled in association with an External Transaction may change on the quarter hour within a given hour.

Section 4.4.1.4:  This section sets forth the basic rules governing RTC’s role in making scheduling decisions for External Transactions.  The proposed modifications specify the timing for RTC to schedule hourly and 15 minute External Transactions at Variably Scheduled Proxy Generator Buses.  The changes also require subsequent RTC runs within the hour to revisit prior External Transaction schedules and to schedule the next set of economic 15 minute External Transactions.

Honorable Kimberly D. Bose December 28, 2010

Page 7

Section 4.4.2:  The changes proposed here are intended to implement five minute 
transactions.  Modifications to subsection 4.4.2.1 specify that the Real-Time Dispatch 
will produce such schedules for External Transactions at Dynamically Scheduled 
Proxy Generator Buses.  New subsection 4.4.2.2 provides that RTD will schedule 
External Transactions on a five minute basis at those locations and authorizes the 
NYISO to change the schedule for energy associated with an External Transaction 
every five minutes.  The new language also clarifies that the existing rules in 
Attachment J of the OATT will apply to External Bilateral Transaction Schedules. 
The subsequent sections are renumbered to accommodate this new subsection.

Section 4.4.3:  The existing section authorizes the NYISO to activate the RTD-CAM software when unexpected system conditions occur.  The proposed changes will permit RTD-CAM to schedule five minute transactions at Dynamically Scheduled Proxy Generator Buses.

Sections 4.5.3.2 and 4.5.4.2:  The proposals for this subsection modify existing rules for the assessment of Financial Impact Charges related to failed transactions (injections and withdrawals) and calculating Curtailment Guarantee Payments to accommodate intra-hour External Transaction scheduling.

Attachment B, Section 17.1:  The NYISO proposes numerous modifications to the 
pricing rules used in calculating real-time LBMPs at Proxy Generator Buses and 
designated Scheduled Lines.  Subsection 17.1.6.1 defines the types of constraints that 
may be binding at a given Proxy Generator Bus and thus dictate application of a 
special pricing rule for an External Transaction.  Changes to the next section, 
renumbered 17.1.6.2, establish the general rules for pricing External Transactions at 
Proxy Generator Buses that are subject to one or more of the defined constraints. 
Different  formulas  apply  depending  on  whether  the  particular  bus  has  been 
designated for hourly, fifteen-minute, or five-minute scheduling.  If the Proxy 
Generator Bus is non-competitive, subsection 6.3 specifies the pricing rules that will 
apply to Transactions at that interface.  A third set of pricing formulas, described at 
subsection 17.1.6.4 applies to Proxy Generator Buses associated with designated 
Scheduled Lines, which may also be subject to any of the defined constraints. 
Finally, subsection 17.1.6.5 will be modified to specify the method for calculating the 
Marginal Loss and Congestion components of real-time LBMPs for Transactions at 
Proxy Generator Buses that are scheduled intra-hour.

Attachment C, Section 18.6.1.2:  This section addresses Suppliers’ eligibility for real-
time Bid Production Cost guarantee payments.  The NYISO will add subparagraph

2.1 to preclude hourly import transactions scheduled at either Variably Scheduled or 
Dynamically Scheduled Proxy Generator Buses from receiving real-time shortfall 
payments.  Similarly, customers will not be eligible for such payments with respect to 
import transactions at Non-Competitive Proxy Generator Buses or at any of the

Honorable Kimberly D. Bose December 28, 2010

Page 8

designated Scheduled Lines, when the facility is export constrained.  Other changes substitute the term “Customer” for “External Generators and other Suppliers.”

Attachment J, Section 25.6.1:  This proposed modification confirms the method for 
determining a customer’s eligibility for an Import Curtailment Guarantee Payment, 
and clarifies the formula that the NYISO will use to calculate an eligible Supplier’s 
payment.

C. 	Changes to the OATT.  The NIYSO proposes corresponding changes to the OATT.

Section 3.1.8:  A minor change to this section recognizes that External Transaction schedules  may  not  be  hourly  and  authorizes  the  NYISO  to  schedule  Firm Transmission Service intra-hourly in real-time.

Attachment J, Section 16.3:  The NYISO will add a new provision that parallels the 
proposed changes to Section 4.4.1 of the Services Tariff.  The new language

authorizes the NYISO to vary External Transaction Schedules more frequently than once an hour, so long as the bidder has opted for variable scheduling.  Transmission Customers will also be allowed to modify Bilateral Transactions in the Real-Time Market.  Additional changes provide the NYISO with needed flexibility to implement intra-hourly scheduling for these transactions.

IV. Effective Date and Implementation Plan

The NYISO requests an effective date of March 15, 2011 for the proposed amendments. The changes to the import curtailment guarantee settlement rules will become effective for all Proxy Generator Buses on that date.  The NYISO intends to phase in the new scheduling and pricing rules on an interface-by-interface basis.  The first location for more frequent Transaction scheduling will be the Hydro-Quebec/Chateauguay interface starting after the requested effective date in March 2011; the specific start date will depend on system operating conditions in both regions.  The NYISO plans to extend implementation to Proxy Generator Buses at the PJM interface in the 4th quarter of 2011, and to the ISO-NE interface in early 2013.  The NYISO will inform its Market Participants of the availability of intra-hour scheduling at a Proxy Generator Bus at least two weeks prior to implementing the change.

V.	Stakeholder Approval

The amendments proposed in this filing were discussed and reviewed five times with 
stakeholders in the Market Issues Working Group forum.  These amendments were unanimously 
approved by the NYISO’s Business Issues Committee on October 6, 2010 and were unanimously 
approved, with abstentions, by the NYISO’s Management Committee on October 21, 2010.

Honorable Kimberly D. Bose December 28, 2010

Page 9

VI.	Communications and Correspondence

All communications and service in this proceeding should be directed to:

Robert E. Fernandez, General Counsel	*Elizabeth A. Grisaru

Ray Stalter	Whiteman Osterman & Hanna

Director of Regulatory Affairs	One Commerce Plaza

*Mollie Lampi	Albany, New York 12260

Assistant General Counsel	Tel:  (518) 487-7624

10 Krey Boulevard	Fax:  (518) 487-7777

Rensselaer, NY 12144	egrisaru@woh.com

Tel:  (518) 356-7530 
Fax: (518) 356-7678 
rfernandez@nyiso.com 
rstalter@nyiso.com 
mlampi@nyiso.com

*Persons designated for receipt of service.

VII.   Service

The NYISO will electronically send a link to this filing to the official representative of 
each of its customers, to each participant on its stakeholder committees, to the New York Public 
Service Commission, and to the electric utility regulatory agencies of New Jersey and 
Pennsylvania.  In addition, the complete filing will be posted on the NYISO’s website at 
www.nyiso.com.  To the extent necessary, the NYISO requests waiver of the requirements of 
Section 35.2(d) of the Commission’s Regulations (18 C.F.R. § 35.2(d) (2007)) to permit it to provide service in this manner.

Honorable Kimberly D. Bose December 28, 2010

Page 10

VIII.  Conclusion

Wherefore, for the foregoing reasons, the New York Independent System Operator, Inc. respectfully requests that the Commission accept for filing the proposed tariff revisions attached hereto with an effective date of March 15, 2011.

Respectfully submitted,

/s/  Elizabeth A. Grisaru

Elizabeth A. Grisaru

Whiteman Osterman & Hanna

Counsel to the New York Independent System Operator, Inc.

One Commerce Plaza

Albany, New York 12260

mailto:egrisaru@woh.com

cc:	Michael A. Bardee

Gregory Berson 
Connie Caldwell 
Anna Cochrane 
Lance Hinrichs 
Jeffrey Honeycutt 
Michael Mclaughlin 
Kathleen E. Nieman 
Rachel Spiker

John Yakobitis