134 FERC ¶61,255

UNITED STATES OF AMERICA

FEDERAL ENERGY REGULATORY COMMISSION

Before Commissioners:  Jon Wellinghoff, Chairman;

Marc Spitzer, Philip D. Moeller,

John R. Norris, and Cheryl A. LaFleur.

New York Independent System Operator, Inc.	Docket Nos.  ER11-2048-000

ER11-2048-001 
ER11-2048-002

ORDER CONDITIONALLY ACCEPTING COMPLIANCE FILING IN PART, 
	REJECTING COMPLIANCE FILING IN PART,

AND REQUIRING ADDITIONAL COMPLIANCE FILING

(March 31, 2011)

1. 	In this order, the Commission accepts in part and rejects in part a filing by the

New York Independent System Operator, Inc. (NYISO) of revisions to its tariff to change 
the method by which NYISO calculates Available Transmission Capacity (ATC).  For 
the reasons discussed below, we accept NYISO's filing in part, as being in partial 
compliance with Order Nos. 729 and 890, effective as of April 1, 2011.  We also direct 
NYISO to submit an additional compliance filing within 60 days of the date of this order.

I.	Background

A.	Order Nos. 729 and 890

2.	On November 24, 2009, the Commission issued Order No. 729, approving six

"Modeling, Data and Analysis" (MOD) Reliability Standards submitted by the North

American Electric Reliability Corporation (NERC).1  The six MOD Reliability Standards 
require certain users, owners, and operators of the Bulk-Power System to develop 
consistent methodologies for the calculation of Available Transfer Capability (ATC) and

1 Mandatory Reliability Standards for the Calculation of Available Transfer

Capability, Capacity Benefit Margins, Transmission Reliability Margins, Total Transfer Capability, and Existing Transmission Commitments and Mandatory Reliability 
Standards for the Bulk-Power System, Order No. 729, 129 FERC ¶ 61,155 (2009), order on reh'g, Order No. 729-A, 131 FERC ¶ 61,109, order on reh'g and reconsideration, Order No. 729-B, 132 FERC ¶ 61,027 (2010).

Docket No. ER11-2048-000, et al.	- 2 -

Available Flowgate Capability (AFC).  Reliability Standard MOD-001-1 "requires

applicable entities to calculate available transfer capability on a consistent schedule and for specific timeframes."2  In Order No. 729-B, the Commission also clarified the 
effective date for the MOD Reliability Standards, stating that the MOD Reliability 
Standards shall become effective on the first day of the first quarter occurring 365 days following approval by the Commission, i.e., April 1, 2011.3

3. 	The Commission also issued Order No. 890, in which it allowed transmission 
providers to propose non-rate terms and conditions that differ from those in Order No. 
890 if those provisions are consistent with or superior to the transmission provider's pro 
forma Open Access Transmission Tariff (OATT).4  To the extent that deviations from the 
pro forma OATT are necessary, the Commission has found such deviations to be just and 
reasonable if the filing party explains how the deviations in the proposed OATT are 
consistent with or superior to the pro forma OATT, or fully explains how the pro forma 
provisions are not applicable given the filing party's business model.5  The Commission 
also stated in Order No. 890 that it required transmission providers to file a revised 
Attachment C to incorporate any changes in NERC’s and the North American Energy 
Standards Board’s (NAESB) revised reliability standards and business practices related to 
ATC calculations.6

2 Order No. 729, 129 FERC ¶ 61,155 at P 20.

3 On September 16, 2010, the Commission issued an order accepting an

interpretation of Reliability Standards MOD-001-1 and MOD-029-1.  North American

Electric Reliability Corporation, 132 FERC ¶ 61,239 (2010) (September 16, 2010 Order). To reflect this interpretation, which is now appended to both standards, these Reliability Standards are now MOD-001-1a and MOD-029-1a.

4 Preventing Undue Discrimination and Preference in Transmission Service,

Order No. 890, FERC Stats. & Regs. ¶ 31,241 at P 135, order on reh'g, Order No. 890-A, FERC Stats. & Regs. ¶ 31,261 (2007), order on reh'g, Order No. 890-B, 123 FERC ¶ 61,299 (2008), order on reh'g, Order No. 890-C, 126 FERC ¶ 61,228 (2009), order on clarification, Order No. 890-D, 129 FERC ¶ 61,126 (2009).

5 Montana Alberta Tie Ltd., 116 FERC ¶ 61,071, at P 60 (2006).

6 Order No. 890, FERC Stats. & Regs. ¶ 31,241 at P 325.

Docket No. ER11-2048-000, et al.	- 3 -

B.	Prior Related Commission Orders

1.	Order on NERC Interpretation of MOD Reliability Standards

4.	On December 2, 2009, NERC petitioned the Commission to approve an

interpretation by NERC of certain MOD Reliability Standards that had been requested by 
NYISO.7  In that proceeding, NYISO describes its ATC calculation as "advisory," and 
thus as different from the "physical reservation" transmission model contemplated by the 
Order No. 890 pro forma OATT.8  NYISO asked for clarification as to whether the 
"advisory ATC" used under the NYISO tariff is subject to the ATC calculation and 
recalculation requirements in MOD-001-1 (now MOD-001-1a) Requirements R2 and R8, 
which specify the methodology and frequency with which a transmission provider must 
calculate ATC or AFC.9  NYISO also asked for clarification as to whether it is necessary

7 See September 16, 2010 Order, 132 FERC ¶ 61,239.

8 NYISO's January 8, 2010 comments in Docket No. RD10-5-000 at 2-3; see also NYISO's November 8, 2010 transmittal letter in this proceeding at 2.

9 Requirements R2 and R8 provide as follows:

R2. Each Transmission Service Provider shall calculate 
[Available Transmission Capacity (ATC)] or [Available 
Flowgate Capacity (AFC)] values as listed below using the 
methodology or methodologies selected by its Transmission 
Operator(s):

R2.1  Hourly values for at least the next 48 hours.

R2.2  Daily values for at least the next 31 calendar 
days.

R2.3  Monthly values for at least the next 12 months (months 2-13).

R8. Each Transmission Service Provider that calculates ATC shall recalculate ATC at a minimum on the following 
frequency, unless none of the calculated values identified in the ATC equation have changed:

R8.1  Hourly values, once per hour. Transmission

Service Providers are allowed up to 175 hours per

(continued)

Docket No. ER11-2048-000, et al.	- 4 -

to document the frequency of "advisory" calculations in the responsible entity's Available Transfer Capacity Implement Document.

5. 	As relevant here, in issuing that interpretation, NERC found that NYISO has

multiple ATC Paths that NERC considers to be subject to the calculation and

recalculation requirements of MOD-001-1 (now MOD-001-1a) Requirements R2 and

R8.10  NERC further found that "advisory ATC" is defined in the NYISO tariff in the

same manner in which NERC defines ATC, thus making it difficult for NERC to

conclude that NYISO's advisory ATC is not the same as ATC as defined by NERC.11

Thus, in its order, the Commission concluded that NYISO must comply with MOD-001-1 
Requirements R2 and R8 to calculate and recalculate ATC for those paths that fit within 
the NERC definition of ATC Paths.12  In reaching this determination, the Commission 
specifically noted that NYISO's external interfaces appear to be ATC Paths, because, 
whether or not ATC is calculated by NYISO, they are "a control area to control area 
interconnection" within the Posted Path definition.13  Moreover, the Commission also 
found that to the extent needed for compliance with MOD-001-1 (now MOD-001-1a), 
NYISO would have to calculate internal flows in order to fulfill its obligation to calculate 
external flows.14

2.	Orders on Request for Waiver of OASIS Posting Requirements

6.	In a separate proceeding, on February 12, 2010, the Commission granted in part

and deferred in part a request filed by NYISO for waivers from certain Open-Access

calendar year during which calculations are not

required to be performed, despite a change in a 
calculated value identified in the ATC equation.

R8.2  Daily values, once per day.

R8.3  Monthly values, once per week.

10 September 16, 2010 Order, 132 FERC 61,239 at P 22.

11 Id.  NERC defines ATC Paths as:  "Any combination of Point of Receipt and Point of Delivery for which ATC is calculated; any Posted Path."

12 Id.

13 Id. P 24.

14 Id. P 26.

Docket No. ER11-2048-000, et al.	- 5 -

Same Time Information Systems (OASIS) posting requirements.15  In the February 12, 
2010 Order, the Commission found that most of the posting regulations at issue were 
incompatible with the transmission services provided under NYISO's current tariff and 
granted the requested waivers.  However, the Commission found that NYISO's request 
for waiver from certain OASIS posting requirements was closely correlated with the 
outcome of NERC's request for Commission approval of its interpretation of the MOD

Reliability Standards, then pending before the Commission.  Therefore, the Commission deferred action on the request for waiver of the Commission's regulations.16

7. 	In light of the September 16, 2010 Order, NYISO submitted a motion on October 
12, 2010, to amend its waiver request to apply only to its internal interfaces.  NYISO 
stated that, given the Commission's guidance in the September 16, 2010 Order, it no 
longer seeks waiver with respect to external interfaces.  The Commission granted 
NYISO's amended request for waiver on December 10, 2010.17  It stated that it was doing 
so subject to the clarification in paragraph 26 of the September 16, 2010 Order, which 
provided that, to the extent needed for compliance with MOD-001-1 of the NERC 
Reliability Standards, NYISO should account for the impacts of its internal congestion on 
its external ATC Paths as accurately as possible, and, to the extent that NYISO has to 
calculate internal flows in order to fulfill its obligation to calculate external flows, it was 
required to do so.18

II.	Description of Filing

8.	On November 8, 2010, as amended on January 4, 2011, and January 7, 2011,

NYISO submitted revised tariff sheets amending Attachment C - Methodology to Assess 
Available Transfer Capability of the NYISO tariff in compliance with Order Nos. 729-B 
and 890.

9. 	NYISO states that its proposed tariff revisions amend its existing Attachment C 
provisions to bring NYISO into full compliance with NERC's Reliability Standards

15 New York Independent System Operator, 130 FERC ¶ 61,104 (2010) (February 12, 2010 Order).

16 Specifically the Commission deferred action on the request for waiver of

18 C.F.R. §§ 37.6(b)(1), 37.6(b)(2)(i-ii), and 37.6(b)(3)(i-iii) (2010).

17 New York Independent System Operator, 133 FERC ¶ 61,208 (2010) (December 10, 2010 Order).

18 December 10, 2010 Order, 133 FERC ¶ 61,208 at P 12-13, citing to September 16, 2010 Order, 132 FERC ¶ 61,239 at P 26.

Docket No. ER11-2048-000, et al.	- 6 -

MOD-004-1, MOD-008-1, and MOD-029-1a.  NYISO states that it proposes only two 
minor variations from NERC's standard language.  First, NYISO has included language 
clarifying that Transmission Flow Utilization will be accounted for under the standard 
MOD-029-1a algorithm for computing Existing Transmission Commitments as part of 
the Other Services variable.  Second, NYISO proposes to replace references to ATC 
Paths in the MOD-029-1a language with references to Interfaces.  NYISO explains that 
its proposed terminology is more inclusive because it encompasses all of the Internal

Interfaces - which are not ATC Paths - and the External Interfaces - which appear to be 
ATC Paths, for which the NYISO will calculate ATC.  Additionally, NYISO's proposed 
tariff provisions state at Section 9.1 that "ATC shall be calculated and posted after the 
close of the ISO’s Day-Ahead Market and Real-Time Market for all Internal and External 
Interfaces and for Scheduled Lines."  NYISO concludes that its variations reflect 
NYISO's financial reservation model, and are consistent with the September 16, 2010 
Order.

10. 	In addition, NYISO'S submittal includes proposed corrections to certain

ministerial errors and omissions.  The revisions:  (i) add language to Section 9.1

clarifying that ATC for Scheduled Lines may be calculated by other ISOs/RTOs that 
control the lines; and (ii) clarify and correct portions of previously accepted tariff 
language in section 9.6, for NYISO's calculation of transmission reliability margin.

11. 	As noted above, NYISO explains that it uses a financial reservation transmission service model, which uses differs substantially from the physical reservation transmission model contemplated by the Order No. 890 pro forma OATT.  NYISO notes that the 
Commission has previously granted NYISO waivers from various OASIS posting 
regulations and from related NAESB standards that were inapplicable to NYISO's 
operations and transmission service model in prior filings.19

Notice of Filing

12. 	Notices of NYISO's November 8, 2010, January 4, 2011, and January 7, 2011 
filings were published in the Federal Register, 75 Fed. Reg. 70,230 (2010), and 76 Fed. 
Reg. 2895, 76 Fed. Reg. 3619 (2011), respectively, with interventions and protests due on 
or before November 29, 2010, January 25, 2011, and January 28, 2011.  A motion to 
intervene was filed by the New York Transmission Owners on November 29, 2010.

19 NYISO points to several cases involving waivers of OASIS standards, including New York Independent System Operator, Inc., 127 FERC ¶ 61,005, at P 7 (2009); New 
York Independent System Operator, Inc., 125 FERC ¶ 61,275, at P 15 (2008); New York 
Independent System Operator, Inc., 117 FERC ¶ 61,197, at P 15-17 (2006); and New 
York Independent System Operator, Inc., 133 FERC ¶ 61,246 (2010).  November 8, 2010 transmittal letter at 4 n.9.

Docket No. ER11-2048-000, et al.	- 7 -

III.	Discussion

A.	Procedural Matters

13.	Pursuant to Rule 214 of the Commission's Rules of Practice and Procedure,

18 C.F.R. § 385.214 (2011), the timely, unopposed motion to intervene serves to make

the entity that filed it a party to this proceeding.

B.	Commission Determination

14.	Except as noted below, the Commission accepts NYISO's compliance filing and

other ministerial corrections to Attachment C.

15. 	The Commission also directs NYISO to submit an additional compliance filing within 60 days from the date of this order, addressing the concern detailed below. 
Section 9.1 of NYISO's Attachment C provides a general overview of how the ISO will calculate and post ATC values for its internal and external interfaces and for its scheduled lines.  Section 9.2 of NYISO's Attachment C provides a methodology for computing firm and non-firm ATC.  Section 9.1 states in part:

ATC shall be calculated and posted after the close of the 
ISO's Day-Ahead Market and Real-Time Market for all 
Internal and External Interfaces and for Scheduled Lines.

Section 9.2 states in part:

These Firm ATC values shall be posted after the close of the Day-Ahead Market and Real-Time Market for all Interfaces and Scheduled Lines.

16. 	The instant filing does not meet NYISO's compliance obligation in P 325 of

Order No. 890 because Sections 9.1 and 9.2 do not include an accurate documentation of 
processes and procedures related to the calculation of ATC.20  In particular, as the 
Commission stated previously, "the NYISO OATT's definition of 'ATC' . . . does not 
fully reflect the 'advisory' nature of most NYISO ATC calculation,"21 and Attachment C 
of the NYISO tariff still does not accurately reflect its method of ATC calculation.  In its

20 As the Commission also noted in the September 16, 2010 Order, 132 FERC ¶ 61,239 at P 20, "[t]o the extent that NYISO's tariff does not reflect its actual mode of operation, . . . NYISO is required to take steps to ensure that its tariff and its actual operations are in harmony."

21 September 16, 2010 Order, 132 FERC ¶ 61,239 at P 20.

Docket No. ER11-2048-000, et al.	- 8 -

filing, NYISO states that "[i]f the posted ATC value for an Interface is zero that is an 
indication that the Interface is congested" and that NYISO may still be able to provide 
firm transmission service over internal interfaces by redispatching its system, if 
transmission customers are willing to pay congestion charges (Section 9.1).  However, 
this statement, without more, does not provide sufficient documentation of the processes and procedures by which NYISO calculates ATC.  We, therefore, require NYISO to 
place into Attachment C of its tariff the processes and procedures with which it calculates ATC, and to provide a current link to the location of the detailed algorithms on its 
website, as required by P 325 of Order No. 890.22

17. 	Additionally, the Commission has made clear that NYISO must comply with 
MOD-001-1 (now MOD-001-1a) Requirements R2 and R8 to calculate and recalculate 
ATC for those paths that fit within the NERC definition of ATC Paths.23  As noted, 
above, although NYISO previously requested waiver from certain OASIS posting 
requirements requiring the calculation and posting of ATC on its internal and external 
interfaces, NYISO acknowledges in this filing, by replacing references to “ATC Paths” in 
the MOD-029-1 (now MOD-029-1a) language with references to “Interfaces,” that it no 
longer seeks waiver of MOD-001-1 (now MOD-001-1a) Requirements R2 and R8 with 
respect to external interfaces.24  Thus, NYISO acknowledges that it should account for 
the impacts of congestion on its external ATC Paths as accurately as possible.  Moreover, 
with regard to transactions on external interfaces, NYISO must meet the timing 
requirements of MOD-001-1a.  Requirements R2 and R8 of MOD-001-1a that require all 
transmission service providers to calculate and recalculate hourly, daily, and monthly 
ATC values.  The proposed tariff provisions filed by NYISO do not specify how these 
hourly, daily and monthly ATC values will be calculated.  Proposed Section 9.1 also 
implies that NYISO does not calculate ATC at the frequency required in R8 of MOD-
001-1a.  By combining external interfaces with internal interfaces, and proposing to 
continue to calculate ATC after the fact, NYISO's proposed tariff provisions do not 
comply with R2 and R8 of MOD-001-1a.  We will, therefore, require NYISO to revise its 
proposed tariff provisions to comply with R2 and R8 of MOD-001-1a in this regard.

22 We further remind NYISO, and other RTOs, that website links must be

maintained and kept up to date in order to remain compliant with Order No. 890.  See

Order No. 890, FERC Stats. & Regs. ¶ 31,241 at P 325 ("Attachment C must provide an accurate documentation of processes and procedures related to the calculation of ATC, not the actual mathematical algorithms themselves, which should be posted on the 
transmission provider's Web site").

23 September 16, 2010 Order, 132 FERC ¶ 61,239 at P 22.

24 Transmittal at page 5.

Docket No. ER11-2048-000, et al.	- 9 -

18.	Finally, we note that MOD-001-1a Requirements R6 and R7 remain unaddressed

in NYISO's Attachment C.25

19. 	Therefore, we direct NYISO to file, within 60 days of the issuance of this order, a 
further compliance filing that either revises sections 9.1 and 9.2 of Attachment C to 
comply with MOD-001-1a Requirements R2 and R8, or explains how its tariff currently 
reflects its actual mode of operation in a way that is consistent with or superior to the 
transmission provider's pro forma Open Access Transmission Tariff (OATT), as required 
by Order No. 890.  We will require NYISO to include a tariff amendment that will update 
or clarify the frequency with which NYISO calculates ATC.  We also require NYISO to 
either revise or explain the calculation assumptions used to determine its Total Transfer 
Capability and ATC values, in compliance with MOD-001-1a Requirements R6 and R7.

20. 	Lastly, the Commission directs NYISO to include in its compliance filing a 
clarification of how NYISO's ATC values are calculated in coordination with other 
transmission providers.  As explained in Order No. 890, transmission providers are 
required to coordinate the calculation of ATC with others and this requires a standard 
means of exchanging data.  The data shall, at a minimum, be exchanged among 
transmission providers for the purposes of ATC modeling and ATC recalculation 
frequency and times.26  Therefore, the Commission directs NYISO to provide a 
description of how it responds to requests for data even when such requests are made 
more frequently than the transmission service provider updates its available transfer or 
flowgate capability model.  Further NYISO should explain its calculation frequency and 
how it adjusts its calculations for neighboring control areas, in its revised Attachment C 
filing.

25 MOD-001-1a Requirements R6 and R7 provide as follows:

R6. When calculating Total Transfer Capability (TTC) or

Total Flowgate Capability (TFC) the Transmission Operator shall use assumptions no more limiting than those used in the planning of operations for the corresponding time period 
studied, providing such planning of operations has been 
performed for that time period.

R7. When calculating ATC or AFC the Transmission Service Provider shall use assumptions no more limiting than those used in the planning of operations for the corresponding time period studied, providing such planning of operations has been performed for that time period.

26 Order No. 890, FERC Stats. & Regs. ¶ 31,241 at P 310.

Docket No. ER11-2048-000, et al.	- 10 -

The Commission orders:

(A)    NYISO's compliance filing is hereby conditionally accepted, as discussed in the body of this order.

(B)    NYISO is hereby required to submit a compliance filing within 60 days from the date of this order, as discussed in the body of this order.

By the Commission.

( S E A L )

Kimberly D. Bose, 
	Secretary.