## 35.20	Schedules

### Schedule A - Description Of Interconnection Facilities

The NYISO – PJM Coordination Agreement covers the PJM – NYISO Interconnection Facilities under the Operational Control of the NYISO and PJM.  For Operational Control purposes, the point of demarcation for each of the Interconnection Facilities listed below is the point at which each Interconnection Facility crosses the PJM-New York State boundary, except as noted below.

The PJM-NYISO Interconnection contains twenty-three (23) alternating current (“AC”) Interconnection Facilities, seven (7) of which form one (1) AC pseudo-tie; and further contains one (1) HVDC Interconnection Facility. These are tabulated below:

NY/PJM AC Interconnection Facilities:

PJM	NYISO	Designated   	(kV)	Common Meter Point

Branchburg	Ramapo	5018	500	Ramapo

Closter 	Sparkill	751	69	Closter

E. Sayre 	N. Waverly	956	115	E. Sayre

E. Towanda 	Hillside	70	230	Hillside

Erie East 	South Ripley	69	230	South Ripley

Franklin	Sugar Loaf	SJ	115	Sugar Loaf

Franklin	Sugar Loaf	SD	115	Sugar Loaf

Harings Corners	Burns	702	138 	Harings

Harings Corners 	Nanuet	45	34	Harings

Harings Corners	W. Nyak	701	69	Harings

Homer City 	Watercure	30	345	Homer

Homer City	Stolle Road	37	345	Homer

Hudson 	Farragut	C3403	345	Farragut

Hudson 	Farragut	B3402	345	Farragut

Linden 	Goethals	A2253	230	Goethals

Montvale 	Pearl River	491	69	Montvale

Montvale 	Blue Hill	44	69	Montvale

Montvale 	Blue Hill	43	69	Montvale

S. Mahwah 	Hilburn	65	69	S. Mahwah

S. Mahwah 	S. Mahwah	138/345	138/345	S. Mahway

S. Mahwah 	Ramapo	51	138	S. Mahwah

Tiffany 	Goudey	952	115	Goudey

Warren 	Falconer	171	115	Warren

RECO	NYISO	AC Pseudo-Tie	Various	O&R EMS

Sayerville	Newbridge	HVDC-Tie	500	New Bridge

### Schedule B - Other Existing Agreements:

1.0	Lake Erie Emergency Redispatch (LEER)

2.0	RAMAPO PHASE ANGLE REGULATOR OPERATING PROCEDURE prepared by the NYPP/PJM Circulation Study Operating Committee

3.0	Operating Protocol for the Implementation of Commission Opinion No. 476, Docket No. EL02-23-000 (Phase II), New York Independent System Operator, Inc., FERC Electric Tariff, Original Vol. No. 2, Attachment M-1.

4.0	Northeastern ISO/RTO Coordination of Planning Protocol

5.0	Inter Control Area Transaction Agreement

6.0	Procedures to Protect for Loss of Phase II Imports (effective January 16, 2007, pursuant to Order issued January 12, 2007, in FERC Docket No. ER07-231-000).

7.0	Unscheduled Transmission Service Agreement, PJM Interconnection L.L.C, Rate Schedule No. 30, Effective Date January 1, 2001.

### Schedule C - Operating Protocol for the Implementation Of Con Ed – PJM Transmission Service Agreements

1.1	This “Operating Protocol” establishes procedures for the planning, operation, control, and scheduling of energy between the New York Independent System Operator, Inc. (“NYISO”) and PJM Interconnection, L.L.C. (“PJM”) (collectively, the “Parties”), associated with two Long-term Firm Point-to-Point Transmission Service Agreements (“TSAs”) entered into by Consolidated Edison Company of New York (“ConEd”) and PJM, dated April 18, 2008, executed in connection with the rollover of contracts dated May 22, 1975 (as amended May 9, 1978) and May 8, 1978 between ConEd and Public Service Electric & Gas Company (“PSE&G”).  The TSA designated Original Service Agreement No. 1874 is referred to herein as the 400 MW transaction and the TSA designated Original Service Agreement No. 1873 is referred to as the 600 MW transaction.  The two contracts are referred to collectively as the “600/400 MW transactions.”

1.1.1	The 400 MW transaction.  The 400 MW transaction has the same level of firmness as other firm transactions, except as provided in section 1.3 of this Operating Protocol.

1.1.2	The 600 MW transaction.  The 600 MW transaction shall have the same level of firmness as other firm transactions.

1.2	This Operating Protocol shall be used by the NYISO and PJM in preparing to operate, and operating in real-time, to the hourly flow of energy between them pursuant to the 600/400 MW transactions as established by this Operating Protocol.

1.3	During system emergencies, the appropriate emergency procedures of the NYISO and PJM, if necessary, shall take priority over the provisions of this Operating Protocol.  The NYISO and PJM shall have the authority to implement their respective emergency procedures in whatever order is required to ensure overall system reliability.  Without limiting the foregoing, the order of load relief measures and transaction reductions when there is an emergency in the PJM Mid-Atlantic Area will be:

• Calling of Emergency Load Response

• Voltage reduction

• Pro-rata load shed and reduction of the 600/400 MW transactions

In addition, if PJM declares an emergency condition that arises from outages on the PSE&G system, the NYISO and PJM may agree to deliver up to 400 MW to Goethals for re-delivery to Hudson via the NYISO’s system.  Such emergency re-deliveries shall not be considered in the calculation of the Real-Time Market Desired Flow under Appendices 1 and 3 of this Operating Protocol.

1.4	All aspects of this Operating Protocol are subject to the dispute resolution procedures set forth in the Joint Operating Agreement Among and Between New York Independent System Operator, Inc., and PJM Interconnection, L.L.C.

1.5	The Parties will review all aspects of this Operating Protocol annually.

1.6	Attached and included as part of this Operating Protocol are the following appendices:  Appendix 1 – Process Flow, Appendix 2 – Transmission Constraints and Outages Associated with the Contracts, Appendix 3 – The Day-Ahead Market and Real-Time Market Desired Flow Calculation, Appendix 4 – Planning Procedures, Appendix 5 – Operation of the PARs, Appendix 6 – Distribution of Flows Associated with Implementation of Day-Ahead and Real Time Market Desired Flows, Appendix 7 – References, and Appendix 8 – Definitions.

WEQ-007 “Inadvertent Interchange Payback Standards,” North American Energy Standards Board (NAESB), on-line at www.naesb.org.

In a maximum generation emergency in the PJM Mid-Atlantic Area where PSE&G load needs to be curtailed, the PSE&G load would be curtailed pro-rata with curtailment of the ConEd requested service (and other firm service on the system). But, if NYISO is not also in a capacity emergency, the desired flow on ABC will be reduced by up to 400 MW to the extent necessary to avoid a PSEG load curtailment. ConEd may upgrade the transmission service for the 400 MW transaction to eliminate the reduction of the 400 MW transaction prior to load shed as described above by requesting such upgraded service and funding all necessary transmission upgrades as required by Part II and Part VI of the PJM OATT.  The 600 MW transaction shall be reduced in the same manner as all other firm transactions in PJM.