## 35.21	Schedules A and B

### Schedule A - Description Of Interconnection Facilities

The NYISO – PJM Coordination Agreement covers the PJM – NYISO Interconnection Facilities under the Operational Control of the NYISO and PJM.  For Operational Control purposes, the point of demarcation for each of the Interconnection Facilities listed below is the point at which each Interconnection Facility crosses the PJM-New York State boundary, except as noted below.

The PJM-NYISO Interconnection contains twenty-three (23) alternating current (“AC”) Interconnection Facilities, seven (7) of which form one (1) AC pseudo-tie; and further contains two (2) HVDC Interconnection Facilities as well as one (1) Variable Frequency Transformer (VFT). These are tabulated below:

NY/PJM Interconnection Facilities:

PJM	NYISO	Designated   	(kV)	Common Meter Point

Branchburg	Ramapo	5018	500	Ramapo

Cresskill 	Sparkill	751	69	Cresskill

E. Sayre 	N. Waverly	956	115	E. Sayre

E. Towanda 	Hillside	70	230	Hillside

Erie East 	South Ripley	69	230	South Ripley

Harings Corners	Corporate Drive	703	138	Harings

Harings Corners 	Pearl River	45	34	Harings

Harings Corners	W. Nyack	701	69	Harings

Homer City 	Watercure	30	345	Homer

Homer City	Stolle Road	37	345	Homer

Hudson 	Farragut	C3403	345	Farragut

Hudson 	Farragut	B3402	345	Farragut

Linden 	Goethals	A2253	230	Goethals

Linden VFT	Linden Cogen	VFT	345	Linden VFT

Montvale 	Pearl River	491	69	Montvale

Montvale 	Blue Hill	44	69	Montvale

Montvale 	Blue Hill	43	69	Montvale

S. Mahwah 	Hilburn	65	69	S. Mahwah

S. Mahwah 	S. Mahwah	BK 258	138/345	S. Mahwah

S. Mahwah 	Ramapo	51	138	S. Mahwah

Waldwick	S. Mahwah	J3410	345	Waldwick

Waldwick	S. Mahwah	K3411	345	Waldwick

Tiffany 	Goudey	952	115	Goudey

Warren 	Falconer	171	115	Warren

RECO	NYISO	AC Pseudo-Tie	Various	O&R EMS

Sayerville	Newbridge	HVDC-Tie	500	Newbridge

Bergen	West 49th 	HVDC-Tie Y56	345	Bergen

NY/PJM Interfaces at which NYISO and PJM are Authorized to Consider CTS Interface Bids:

| PJM Interface Name | PNODE ID | Corresponding NYISO Proxy Generator Buses | PTID |
| --- | --- | --- | --- |
| NYIS | 5413134 | PJM_GEN_KEYSTONE | 24065 |
| NYIS | 5413134 | PJM_LOAD_KEYSTONE | 55857 |
| LindenVFT | 81436855 | PJM_GEN_VFT_PROXY | 323633 |
| LindenVFT | 81436855 | PJM_LOAD_VFT_PROXY | 355723 |
| Neptune | 56958967 | PJM_GEN_NEPTUNE_PROXY | 323594 |
| Neptune | 56958967 | PJM_LOAD_NEPTUNE_PROXY | 355615 |
| HudsonTP | 1124361945 | PJM_HTP_GEN | 323702 |
| HudsonTP | 1124361945 | HUDSONTP_345KV_HTP_LOAD | 355839 |

### Schedule B - Other Existing Agreements:

1.0	Lake Erie Emergency Redispatch (LEER)

2.0	RAMAPO PHASE ANGLE REGULATOR OPERATING PROCEDURE prepared by the NYPP/PJM Circulation Study Operating Committee.

3.0	Northeastern ISO/RTO Coordination of Planning Protocol

4.0	Inter Control Area Transaction Agreement.

5.0	Procedures to Protect for Loss of Phase II Imports (effective January 16, 2007, pursuant to Order issued January 12, 2007, in FERC Docket No. ER07-231-000).

6.0	Joint Emergency Operating Protocol dated September 10, 2009, among PJM Interconnection, L.L.C., New York Independent System Operator, Inc., and Linden VFT, LLC (Filed by PJM on October 1, 2009, in FERC Docket No. ER09-996-000).

WEQ-007 “Inadvertent Interchange Payback Standards,” North American Energy Standards Board (NAESB), on-line at www.naesb.org.

See NYISO Market Administration and Control Area Services Tariff Section 4.4.4 for additional information.